CAN_Protocol: Lesson_1

- Le bus de données CAN:

- (Controller Area Network) est un bus système séries très répandu dans beaucoup d'industries, notamment l'automobile.

Il a été normalisé avec la norme ISO11898.

In-vehicle networks (IVN):

are specialized internal communication networks that interconnect various components inside a vehicle. The components inside the vehicle include ECUs, gateways, sensors, actuators, etc.

To coordinate communication among ECUs, In-Vehicle Networks (IVNs) are composed of several kinds of bus protocols.

Evolution of in-vehicle networks:

Historically, in-vehicle networks were organized into logical domains, such as"Body", "Chassis", and "Powertrain".

These domains were interconnected through a central gateway.

*:* In the future, the concept of specialized ECUs fordomain-specific functions will continue, but the general trendis moving toward separation according to physical location (Zones) rather than by logical function.

*:* Zone ECUs connect via high-speed networks to a central ECU where much of the processing is done.

Why CAN Bus ?

From slide 10 to slide 16

Remote Frame is a request with specific ID from a node to tell other nodes that it wants to receive Data of this ID.

CAN Overview:

- Multi-master protocol

- Broadcasting:

refers to the way CAN messages are transmitted across the network. 	Unlike point-to-point communication where messages are sent 	directly from one device to another, broadcasting in CAN means that 	when a device (node) sends a message, it is sent to all other devices 	on the network simultaneously. Each node receives the message and 	decides whether it needs to process it based on the message ID.

- Event-Driven

- Asynchronous communication (Event Triggered)

- Serial communication technology

- Priority-basedbit-wise arbitration

- Variable message priority basedon 11-bit(or extended29 bit) packet identifier